Go to last page for steps to COMPILE

(no need to RUN the code, since there is no hardware)

- BLINK LED

// setup() runs only one time when we start or reset the Arduino

void setup() {

pinMode(13, OUTPUT);   // We tell Arduino that pin number 13 will be used to give output (like turning an LED ON or OFF)

}

// loop() keeps running again and again forever

void loop() {

digitalWrite(13, HIGH);  // We turn ON the LED connected to pin 13

delay(1000);             // Wait for 1 second (1000 milliseconds) so we can see the light ON clearly

digitalWrite(13, LOW);   // We turn OFF the LED connected to pin 13

delay(1000);             // Wait for 1 second before turning it ON again

}

- BUTTON CONTROLLED LED

// We are creating two variables to store pin numbers

int button = 2;    // The button is connected to pin number 2

int led = 13;      // The LED is connected to pin number 13

// setup() runs only one time when we start or reset the Arduino

void setup() {

pinMode(button, INPUT);   // We set pin 2 as INPUT because we will *read* the button’s signal

pinMode(led, OUTPUT);     // We set pin 13 as OUTPUT because we will *send* signal to turn LED ON or OFF

}

// loop() keeps running again and again forever

void loop() {

int state = digitalRead(button);  // Read the button’s current state: HIGH when pressed, LOW when not pressed

digitalWrite(led, state);         // Send the same signal to LED: if button is pressed (HIGH), LED turns ON; if not, it turns OFF

}

- LM35 TEMPERATURE SENSOR

int sensor = A0;      // The temperature sensor is connected to analog pin A0

float temp;           // Variable to store the temperature value

void setup() {

Serial.begin(9600); // Start serial communication at 9600 bits per second

// (so we can see the temperature readings on Serial Monitor)

}

void loop() {

int val = analogRead(sensor);     // Read the analog value from the temperature sensor (range: 0 to 1023)

// Convert the analog reading to temperature in °C

// The formula converts voltage (0–5V) to temperature assuming LM35 sensor

temp = (val * 5.0 * 100) / 1023;

Serial.println(temp);             // Print the temperature value on the Serial Monitor

delay(500);                       // Wait for 0.5 seconds before reading again

}

- TURNED ON LED :

// The setup() function runs once when the Arduino starts or is reset

void setup() {

pinMode(13, OUTPUT);   // We are telling Arduino that pin 13 will be used as an OUTPUT pin (to send signals to an LED)

}

// The loop() function keeps running again and again forever

void loop() {

digitalWrite(13, HIGH);  // Turn ON the LED connected to pin 13 by sending HIGH voltage

}

- ULTRASONIC DISTANCE MEASUREMENT :

int trig = 8;        // The Trig pin of the ultrasonic sensor is connected to Arduino pin 8

int echo = 7;        // The Echo pin of the ultrasonic sensor is connected to Arduino pin 7

long duration;       // Variable to store the time taken for the sound wave to return

float distance;      // Variable to store the calculated distance

void setup() {

Serial.begin(9600);   // Start serial communication so we can see the distance on Serial Monitor

pinMode(trig, OUTPUT); // Set trig pin as OUTPUT — it sends the ultrasonic sound waves

pinMode(echo, INPUT);  // Set echo pin as INPUT — it receives the reflected sound waves

}

void loop() {

// Send a short 10 microsecond pulse to trigger the ultrasonic sensor

digitalWrite(trig, LOW);       // Make sure trig pin is LOW first

delayMicroseconds(2);          // Wait for 2 microseconds for stability

digitalWrite(trig, HIGH);      // Turn trig pin HIGH to send out the sound wave

delayMicroseconds(10);         // Keep it HIGH for 10 microseconds (required trigger time)

digitalWrite(trig, LOW);       // Turn it LOW again after sending the pulse

// Measure how long it takes for the sound wave to return

duration = pulseIn(echo, HIGH);  // Measure the time (in microseconds) that echo pin stays HIGH

// Calculate the distance using the formula: distance = (time × speed of sound) / 2

distance = duration * 0.034 / 2; // 0.034 cm/µs is the speed of sound; divide by 2 (to & fro)

Serial.println(distance);        // Print the distance value on Serial Monitor

delay(500);                      // Wait for half a second before next measurement

}

- TURN LED OFF :

// The setup() function runs once when the Arduino starts or is reset

void setup() {

pinMode(13, OUTPUT);   // We tell the Arduino that pin 13 will be used as an OUTPUT pin (to control an LED or another device)

}

// The loop() function runs again and again forever

void loop() {

digitalWrite(13, LOW);   // Turn OFF the LED connected to pin 13 by sending LOW voltage (no current)

}

- FADE LED (PWM on PIN 9) :

// The setup() function runs once when the Arduino starts or is reset

void setup() {

pinMode(9, OUTPUT);   // Tell Arduino that pin 9 will be used as an OUTPUT (for LED or any output device)

}

// The loop() function keeps running forever

void loop() {

// This loop slowly increases the brightness of the LED

for (int i = 0; i <= 255; i++) {   // i starts from 0 (off) and goes up to 255 (fully bright)

analogWrite(9, i);   // Send a PWM signal to pin 9 to control LED brightness

delay(10);           // Wait 10 milliseconds before increasing brightness a little more

}

// This loop slowly decreases the brightness of the LED

for (int i = 255; i >= 0; i--) {   // i starts from 255 (bright) and goes down to 0 (off)

analogWrite(9, i);   // Send a lower PWM signal each time to dim the LED

delay(10);           // Wait 10 milliseconds before dimming more

}

}

- BUTTON PRESS COUNTER :

// Declare and initialize variables

int button = 2;    // The button is connected to digital pin 2

int count = 0;     // A counter variable to count how many times the button is pressed

// The setup() function runs once when the Arduino starts or is reset

void setup() {

Serial.begin(9600);       // Start serial communication to send data to the computer

pinMode(button, INPUT);   // Set pin 2 as an INPUT pin (to read button state)

}

// The loop() function keeps running forever

void loop() {

// Check if the button is pressed

if (digitalRead(button) == HIGH) {   // Read the button state; HIGH means it’s pressed

count++;                           // Increase the count by 1

Serial.println(count);             // Print the current count value on the Serial Monitor

delay(300);                        // Wait for 300 ms to avoid multiple counts from one press (debouncing)

}

}

- PRINT HELLO :

// The setup() function runs once when the Arduino starts or is reset

void setup() {

Serial.begin(9600);   // Start serial communication at 9600 bits per second (baud rate)

}

// The loop() function keeps running again and again forever

void loop() {

Serial.println("Hello");   // Send the word "Hello" to the Serial Monitor and move to the next line

delay(1000);               // Wait for 1000 milliseconds (1 second) before repeating

}

- BUTTON-ACITIVATED BUZZER :

// Declare variables for components

int button = 2;    // The button is connected to digital pin 2

int buzzer = 8;    // The buzzer is connected to digital pin 8

// The setup() function runs once when the Arduino starts or is reset

void setup() {

pinMode(button, INPUT);    // Set pin 2 as an INPUT (to read button press)

pinMode(buzzer, OUTPUT);   // Set pin 8 as an OUTPUT (to control the buzzer)

}

// The loop() function keeps running again and again forever

void loop() {

int state = digitalRead(button);   // Read the button state (HIGH when pressed, LOW when not pressed)

digitalWrite(buzzer, state);       // Send the same state to the buzzer — turns ON when button is pressed, OFF when released

}

- MULTIPLE LEDS ON :

// The setup() function runs once when the Arduino starts or is reset

void setup() {

pinMode(12, OUTPUT);   // Set pin 12 as an OUTPUT (to control an LED or other device)

pinMode(13, OUTPUT);   // Set pin 13 as an OUTPUT (to control another LED or device)

}

// The loop() function keeps running again and again forever

void loop() {

digitalWrite(12, HIGH);   // Turn ON the device (like an LED) connected to pin 12

digitalWrite(13, HIGH);   // Turn ON the device (like an LED) connected to pin 13

}

- SERIAL PRINT NUMBERS :

// Declare a variable

int i = 0;   // Start counting from 0

// The setup() function runs once when the Arduino starts or is reset

void setup() {

Serial.begin(9600);   // Start serial communication at 9600 bits per second

}

// The loop() function keeps running again and again forever

void loop() {

Serial.println(i);    // Print the current value of i on the Serial Monitor

i++;                  // Increase i by 1 each time (i = i + 1)

delay(1000);          // Wait for 1 second before printing the next number

}

STEPS TO COMPILE :

- Open Arduino IDE

- Go to Tools → Board → Arduino Uno

- Paste your code in the editor

- Click ✔️ Verify (top-left)

- Wait for the message “Done compiling” at the bottom